Platform.IO Wearable Device

Product Requirements Document (PRD)

Version 1.0  |  February 2026  |  CONFIDENTIAL

# 1. Overview

This document defines the product requirements for a wearable IoT device developed using the Platform.IO embedded development environment. The device integrates a comprehensive sensor suite, wireless communication stack, and haptic feedback capabilities to enable health monitoring, identity/access, and intra-body communication use cases.

The development target is the nRF5340 SoC (dual-core ARM Cortex-M33) with a companion development/prototyping board used during the hardware bring-up phase.

# 2. Goals & Non-Goals

## 2.1 Goals

- Deliver a fully functional wearable platform with multi-sensor data acquisition

- Enable real-time biometric and environmental monitoring via BLE

- Provide intra-body communication (IBC) via galvanic coupling as a novel connectivity channel

- Support NFC-based identity and access control

- Develop entirely within the Platform.IO ecosystem for reproducibility and CI integration

## 2.2 Non-Goals

- Cloud backend / data storage infrastructure

- Regulatory certification (CE, FCC) in this phase

- Consumer-ready enclosure design

- Production-scale manufacturing

# 3. Platform & Toolchain

| Component | Selection | Notes |
| --- | --- | --- |
| IDE / Build System | Platform.IO | VS Code extension + CLI; supports Zephyr RTOS |
| Primary SoC | nRF5340 | Dual Cortex-M33; app core (128 MHz) + net core (64 MHz) |
| Dev / Eval Board | nRF5340-DK (or custom) | Used for hardware bring-up and peripheral validation |
| RTOS | Zephyr RTOS (via nRF Connect SDK) | Preferred; FreeRTOS acceptable as fallback |
| Programming Language | C / C++ | Zephyr device tree for hardware abstraction |
| Debugger | J-Link / OpenOCD | Integrated in Platform.IO |

# 4. Hardware Requirements

The device integrates the following sensor and peripheral subsystems:

## 4.1 6-Axis IMU (Accelerometer + Gyroscope)

- Accelerometer: 3-axis, configurable range (±2g to ±16g), low-power mode

- Gyroscope: 3-axis, configurable range (±250 to ±2000 dps)

- Interface: SPI or I2C

- Typical parts: ICM-42688-P, LSM6DSO, BMI270

- Required: FIFO buffering, interrupt-driven wakeup, configurable ODR

## 4.2 Skin Sensor — Capacitive Touch

- Type: Capacitive touch sensing for skin contact detection

- Function: On-body detection, gesture recognition

- Interface: I2C or dedicated capacitive sensing peripheral on nRF5340

- Optional: Galvanic Skin Response (GSR) if combined with IBC electrodes

## 4.3 Vibration Motor (Haptic Feedback)

- Type: ERM (Eccentric Rotating Mass) or LRA (Linear Resonant Actuator)

- Driver: DRV2605L or GPIO + MOSFET driver

- Patterns: Configurable via software (alerts, notifications, IBC signaling confirmation)

## 4.4 USB

- nRF5340 native USB 2.0 Full Speed (12 Mbps)

- Use cases: Firmware update (DFU), serial debug console, USB CDC-ACM

- Power: USB bus power for charging and development

## 4.5 Temperature & Humidity — AHT10

- Sensor: AHT10 (I2C, 3.3V)

- Temperature accuracy: ±0.3°C; Humidity accuracy: ±2% RH

- Use case: Ambient environment monitoring; skin temperature as proxy health metric

- Interface: I2C (address 0x38)

## 4.6 Bluetooth Low Energy (BLE)

- nRF5340 integrated BLE 5.3 radio (net core)

- Profiles: GATT server exposing sensor data characteristics; HID over GATT optional

- Advertising: Configurable intervals; directed advertising for reconnection

- Security: BLE pairing with LESC (LE Secure Connections)

- OTA DFU: nRF Connect SDK SMP server (via mcumgr)

## 4.7 Buttons

- Minimum 1 user-accessible button (GPIO, active-low, internal pull-up)

- Functions: Short press = mode cycle; Long press (3s) = power toggle / factory reset

- Debounce: Software debounce (10ms) required

## 4.8 NFC (Near Field Communication)

- nRF5340 integrated NFC-A tag (passive, Type 2 Tag)

- Use cases: Device pairing handshake, identity credential presentation, asset tracking

- NDEF records: URL record for onboarding; custom application record for credential

- Antenna: External loop antenna (PCB trace or flex)

## 4.9 RGB LED — WS2812B

- Type: WS2812B addressable RGB LED (NeoPixel-compatible)

- Interface: Single-wire protocol via GPIO (PWM / SPI emulation)

- Use cases: Status indication (BLE, charging, alerts), color-coded sensor states

- Driver: Zephyr LED Strip driver or custom SPI bitbang

# 5. Galvanic Coupling — Intra-Body Communication (IBC)

Intra-Body Communication (IBC) via galvanic coupling uses the human body as a conductive medium to transmit data between wearable devices worn on different body parts. This is a key differentiating feature of the platform.

| Parameter | Specification |
| --- | --- |
| Method | Galvanic coupling — differential signal injection/detection via skin electrodes |
| Frequency range | 100 kHz to 2 MHz |
| Modulation | ASK, FSK, or OOK (TBD based on channel characterization) |
| Data rate target | Up to 1 Mbps (depending on channel conditions) |
| Electrodes | Two-electrode differential (TX+/TX−, RX+/RX−) |
| Signal generation | nRF5340 PWM/Timer + external analog front-end |
| Reception | Analog front-end (amplifier + filter) → ADC or comparator |
| Isolation | Coupling capacitors to block DC; AC-coupled differential signal |
| Safety | Signal amplitude <1V peak; current limited; no DC path |
| Use cases | Device-to-device handshake, body area network, SOS relay |

## 5.1 IBC Software Stack

- Physical layer: Timer-driven signal generation on nRF5340 app core

- Frame format: Preamble + device ID + payload + CRC16

- Protocol: Simple request-response; CSMA/CA for multi-device environments

- Platform.IO task: Dedicated IBC Zephyr thread with defined priority

# 6. Software Architecture (Platform.IO / Zephyr)

## 6.1 Project Structure

The project will be structured as a standard Platform.IO + Zephyr project:

- platformio.ini — board target (nrf5340dk_nrf5340_cpuapp), framework (zephyr), lib_deps

- src/main.c — application entry point, Zephyr kernel init

- src/sensors/ — IMU, AHT10, capacitive touch drivers

- src/ble/ — GATT service definitions, advertising, OTA DFU

- src/ibc/ — Galvanic coupling TX/RX driver, protocol stack

- src/haptic/ — Vibration motor patterns

- src/led/ — WS2812B driver and animation state machine

- src/nfc/ — NDEF tag content management

- boards/ — Custom board overlay (.overlay) and device tree additions

## 6.2 RTOS Task Priorities

| Thread | Priority | Stack Size | Interval |
| --- | --- | --- | --- |
| IBC RX | 0 (highest) | 2048 B | Interrupt-driven |
| Sensor Sampling | 2 | 1024 B | 50 ms |
| BLE Notifications | 3 | 1024 B | 100 ms |
| LED / Haptic | 5 | 512 B | On event |
| USB CDC Logger | 7 (lowest) | 1024 B | On demand |

# 7. Functional Requirements

| ID | Requirement | Priority |
| --- | --- | --- |
| FR-01 | Device shall sample IMU at minimum 50 Hz and detect motion events | P0 |
| FR-02 | Device shall detect skin contact via capacitive sensor within 500ms | P0 |
| FR-03 | Device shall read AHT10 temperature and humidity every 10 seconds | P0 |
| FR-04 | Device shall advertise BLE and maintain connection to a central device | P0 |
| FR-05 | Device shall support OTA firmware update over BLE (SMP/mcumgr) | P0 |
| FR-06 | Device shall present an NFC NDEF tag when polled by an NFC reader | P1 |
| FR-07 | Device shall transmit and receive IBC frames in the 100 kHz–2 MHz band | P1 |
| FR-08 | Device shall produce configurable haptic patterns via vibration motor | P1 |
| FR-09 | Device shall indicate status via WS2812B RGB LED | P1 |
| FR-10 | Device shall respond to button press events (short / long) | P0 |
| FR-11 | Device shall log data over USB CDC for debugging | P2 |

# 8. Non-Functional Requirements

- Power: Device shall achieve >8 hours operation on a 200 mAh LiPo in normal mode

- Latency: BLE notification latency shall be <100ms from sensor event

- IBC range: Reliable IBC communication across body (wrist-to-wrist, wrist-to-ankle)

- BLE range: Minimum 10 meters open space

- Build: Firmware shall build cleanly in Platform.IO CI with zero warnings

- Code quality: Static analysis (cppcheck or clang-tidy) integrated in CI pipeline

# 9. Development Board Phase (Bring-Up Plan)

| Milestone | Deliverable | Target |
| --- | --- | --- |
| M1 — Toolchain | Platform.IO + Zephyr builds and flashes to nRF5340-DK | Week 1 |
| M2 — Sensors | IMU, AHT10, capacitive touch reading via I2C/SPI; logged over USB CDC | Week 2–3 |
| M3 — BLE | GATT server advertising sensor data; confirmed connection from mobile app | Week 3–4 |
| M4 — Peripherals | WS2812B LED, vibration motor, NFC tag, button handling | Week 4–5 |
| M5 — IBC | IBC TX/RX demonstrated on dev board with analog front-end breadboard circuit | Week 5–7 |
| M6 — Integration | All subsystems integrated; power profiling; OTA DFU validated | Week 7–8 |
| M7 — Custom PCB | Schematic and layout review; gerbers sent to fab | Week 9–10 |

# 10. Open Questions & Risks

- IBC analog front-end: Which discrete IC or MCU peripheral will generate/detect signals in 100 kHz–2 MHz range? (Candidate: Si5351, nRF5340 PWM + external op-amp)

- Capacitive touch: Will the nRF5340 COMP/LPCOMP peripheral suffice, or is a dedicated capacitive touch IC (e.g., AT42QT) needed?

- IBC coexistence: How will IBC and BLE/NFC coexist without RF interference?

- Battery connector: JST-PH 2-pin vs pogo pins for wearable form factor?

- Skin sensor dual-use: Can IBC electrodes double as GSR / skin conductance sensors?

Platform.IO Wearable PRD  |  v1.0  |  Internal Use Only